온프레미스 AI 풀스택 인프라 & 견적 종합 마스터 제안서

[ Comprehensive Architecture & Budget Master Proposal ]

# 온프레미스 AI 풀스택 인프라 & TCO 종합 제안서

Red Hat OpenShift AI (SNO 공유형) + Dell/NVIDIA GPU + Cohere Suite & Qwen2.5-Coder + SAGE 파이프라인 통합 제안

### 1. 총 5개 단계별 아키텍처 예산 종합 비교표 (VAT 별도)

| 구분 항목 | Option B: 상용 HA (8x H100) | Option A-2: SNO PoC (8x H100) | Option A-1: 하이브리드 (4x Blackwell) | Option C-2: 차세대 코딩 (2x Blackwell) | Option C-1: 코딩 Minimal (2x Ada) |
| --- | --- | --- | --- | --- | --- |
| 핵심 목적 | 상용 대규모 무중단 HA | 성능형 8GPU PoC | 사내 RAG + 코딩 겸용 | 차세대 초고속 코딩전용 | 코딩전용 Ultra-Minimal |
| GPU 서버 체급 | Dell XE9680 1대 | Dell XE9680 1대 | Dell R760xa 1대 | Dell R760 1대 | Dell Workstation 1대 |
| GPU 모델 & VRAM | 8x H100 (640GB) | 8x H100 (640GB) | 4x Blackwell (240GB) | 2x Blackwell (120GB) | 2x Ada 48GB (96GB) |
| Control / Storage | Master 3대 + Storage 3대 | 0대 (SNO 통합) | 0대 (SNO 통합) | 0대 (SNO 통합) | 0대 (SNO 통합) |
| HW 소계 (CAPEX) | 662,850,000 원 | 459,000,000 원 | 125,000,000 원 | 65,000,000 원 | 45,000,000 원 |
| SW 연간 OPEX | 287,550,000 원 / 년 | 267,300,000 원 / 년 | 243,000,000 원 / 년 | 28,350,000 원 / 년 | 28,350,000 원 / 년 |
| AI 모델 라이선스 | Cohere On-prem 포함 | Cohere On-prem 포함 | Cohere On-prem 포함 | 0 원 (Qwen2.5-Coder) | 0 원 (Qwen2.5-Coder) |
| 구축 SI 비용 (1회성) | 130,000,000 원 | 60,000,000 원 | 40,000,000 원 | 30,000,000 원 | 25,000,000 원 |
| 1년 차 총 예산 (TCO) | 1,080,400,000 원 | 786,300,000 원 | 408,000,000 원 | 123,350,000 원 | 98,350,000 원 |

### 2. 핵심 검증: Control Plane과 GPU 노드의 1노드 공유 (Single Node OpenShift SNO)

💡 💡 Red Hat 공식 지원 검증: 단 1대의 물리 서버 내부에서 OpenShift K8s Master Control Plane + GPU Worker + RHOAI Operator + LVM NVMe 스토리지의 통합 공유 운용이 100% 가능하며, 별도 6대의 Master/Storage 노드 구매를 생략하여 초기 구축비를 수억 원 절감합니다.

### 3. 서버 메모리(RAM) & Local NVMe SSD 추천 사양 및 Ceph 대치 솔루션

#### System RAM & NVMe SSD 사양

• RAM: 512GB ~ 1TB DDR5 ECC Registered (SNO Control Plane 32GB+ & vLLM KV Cache & Vector DB 상주)

          • NVMe SSD: 4x 7.68TB Enterprise U.2/U.3 (Raw 30.7TB)

            - 1.92TB RAID1 Mirror: OS Boot & OpenShift etcd

            - 10TB: vLLM / HuggingFace 모델 가중치 초고속 캐시

            - 15TB: OpenShift LVM Storage PV

#### Ceph 대체 로컬/공유 스토리지 솔루션

• OpenShift LVM Storage (topolvm): 단일 서버 로컬 NVMe를 K8s 고성능 CSI PV로 변환하여 Vector DB 및 DB에 초저지연 I/O 공급.

          • MinIO Object Storage: OpenShift SNO 내부 StatefulSet 컨테이너 배포로 S3 API 호환 모델 파일/원시 문서 관리.

### 4. AI 아키텍처 및 GPU 자원 파티셔닝 토폴로지

#### Option A-1: 4x Blackwell 하이브리드 배치

[다이어그램]
flowchart TB subgraph Dell_R760xa ["Dell PowerEdge R760xa (4x Blackwell 60GB = 240GB VRAM)"] G01["GPU 0 & 1 (120GB, TP=2)Cohere Command A / Qwen2.5-Coder-32B"] G2["GPU 2 (60GB VRAM)Cohere Embed v3 / Qwen-14B FIM"] G3["GPU 3 (60GB VRAM)Cohere Reranker v3 + Qdrant Vector DB"] end

#### Option C-1/C-2: 2x GPU 코딩 전용 배치

[다이어그램]
flowchart TB subgraph Dell_R760 ["Dell Server / Workstation (2x GPU = 96GB~120GB VRAM)"] G0["GPU 0 (48GB/60GB)Qwen2.5-Coder-32B (Chat/Refactor FP8)"] G1["GPU 1 (48GB/60GB)Qwen2.5-Coder-14B (FIM Realtime Auto-complete)"] end

### 5. Red Hat OpenShift AI (RHOAI) vLLM & KServe 배포 YAML 스크립트 모음

#### A. Cohere Command vLLM ServingRuntime (`vllm-cohere-runtime.yaml`)

apiVersion: serving.kserve.io/v1alpha1
kind: ServingRuntime
metadata:
  name: vllm-cohere-runtime
  namespace: rhoai-cohere-poc
spec:
  supportedModelFormats:
    - name: vLLM
  containers:
    - name: kserve-container
      image: quay.io/modh/vllm:rhoai-2.14
      command: ["python3", "-m", "vllm.entrypoints.openai.api_server"]
      args:
        - "--model=/mnt/models/cohere-command-r"
        - "--tensor-parallel-size=2"
        - "--quantization=fp8"
        - "--max-model-len=16384"
        - "--gpu-memory-utilization=0.90"
      resources:
        limits: { nvidia.com/gpu: "2", memory: "64Gi" }
        requests: { nvidia.com/gpu: "2", memory: "32Gi" }

#### B. Qwen2.5-Coder-32B vLLM ServingRuntime (`qwen-coder-runtime.yaml`)

apiVersion: serving.kserve.io/v1alpha1
kind: ServingRuntime
metadata:
  name: qwen25-coder-runtime
  namespace: rhoai-coding-agent
spec:
  supportedModelFormats:
    - name: vLLM
  containers:
    - name: kserve-container
      image: quay.io/modh/vllm:rhoai-2.14
      command: ["python3", "-m", "vllm.entrypoints.openai.api_server"]
      args:
        - "--model=Qwen/Qwen2.5-Coder-32B-Instruct"
        - "--tensor-parallel-size=2"
        - "--quantization=fp8"
        - "--max-model-len=32768"
        - "--enable-prefix-caching"
      resources:
        limits: { nvidia.com/gpu: "2", memory: "64Gi" }
        requests: { nvidia.com/gpu: "2", memory: "32Gi" }

### 6. 기업용 SAGE 파이프라인 & 사내 보안 코딩 에이전트 통합 연동

[다이어그램]
flowchart LR subgraph Clients ["Client Applications"] DevIDE["VS Code / JetBrains(Continue.dev Plugin)"] SAGEPortal["SAGE Gateway / Open WebUI"] end subgraph OpenShift_SNO ["Red Hat OpenShift AI (Single Node)"] DevIDE --> Qwen_vLLM["Qwen2.5-Coder vLLM(GPU 0~1 FP8)"] SAGEPortal --> VectorDB["Vector DB (Qdrant)지식/코드 인덱스"] SAGEPortal --> Cohere_LLM["Cohere Command A vLLM(GPU 0~1 FP8)"] VectorDB --> Cohere_Rerank["Cohere Reranker v3(GPU 3)"] Cohere_Rerank --> Cohere_LLM end

────────────────────────────────────────────────────────────

Dell / NVIDIA / Red Hat OpenShift AI Full Stack Master Architecture Proposal © 2026 AI Architecture Engineering Team. All rights reserved.